Impaired Waters Monitoring and Implementation Plan:  2022 Annual Evaluation

Impaired Waters

The following water have been identified as impaired by the latest 305(b)/303(d) list of waters and are the subject of this plan.  In each case, the pollutant of concern is fecal coliform.

- Barber Creek

- Calls Creek

- McNutt Creek

- Middle Oconee River

Monitoring

Oconee County Water Resources staff performed monitoring according to the approved plan:

- Upstream and downstream samples were collected from each of the impaired waters in order to calculate two geometric means for the year.

  - A minimum of four samples were collected (one week apart) from each sampling site during the months of April and October 2022.

  - Laboratory analysis for fecal coliform was conducted and the geometric mean for each site was calculated.  The results are shown in the table below:

|  | April - Upstream | April - Downstream | October - Upstream | October - Downstream |
| --- | --- | --- | --- | --- |
| Barber Creek | 236.29 | 205.44 | 291.57 | 174.47 |
| Calls Creek | 854.00 | 405.95 | 393.00 | 104.87 |
| McNutt Creek | 741.45 | 261.10 | 166.76 | 320.54 |
| Middle Oconee River | 83.71 | 87.47 | 214.34 | 322.77 |

Trend in Water Quality

The following graphs compare data results over time for each of the four impaired waters:

During calendar year 2022, the upstream sampling of Barber Creek was higher than the downstream sampling.  Over the sampling period from 2016, the downstream sampling has historically been higher than the upstream.  Calendar year 2022 showed improvements in the stream.  It is recommended to continue monitoring the stream to see if further improvements are documented.

Based on the sampling results since 2016, the downstream sampling of Calls Creek is consistently lower than the upstream sampling.  This data suggests Calls Creek is improving from the upstream sampling point to the downstream sampling point.

Based on the sampling results since 2016, the downstream sampling of McNutt Creek is consistently lower than the upstream sampling.  However, in calendar year 2022, the October sampling showed the downstream higher than the upstream samples.  This data suggests McNutt Creek is remaining stationary and may be improving from the upstream sampling point to the downstream sampling point.

Based on the sampling results since 2016, the downstream sampling of the Middle Oconee River has been higher than the upstream sampling in six of 13 samples.  This data has remained fairly stationary over the last two years.  The sampling data for calendar year 2022 showed the downstream results were higher than the upstream results.  The river is trending to remaining stationary from degrading during the early years of this permit period.

Evaluation

BMP’s included from the County’s SWMP

1.  Public Education and Outreach BMP 2:  Website

2.  Public Education and Outreach BMP 3:  Social Media Program

3.  Public Education and Outreach BMP 4:  Distribution of educational material at events attended by Keep Oconee Beautiful

4.  Public Involvement/Participation BMP 2:  River cleanup event by Keep Oconee Beautiful

5.  Illicit Discharge Detection and Elimination BMP 1:  Ordinance to prohibit illicit discharges to the storm drain system

6.  Illicit Discharge Detection and Elimination BMP 3:  Dry weather screenings

Additional BMP’s

7.  Pet waste stations at public parks

8.  Inspection priority of areas upstream of impaired waters

Some of the waters addressed in this plan enter into Oconee County already impaired, but the downstream sampling does not indicate a higher level of the pollutant in some cases.  Public education has been targeted more specifically to address the source of impairment (fecal coliform) in order to try to further reduce the pollutant.  It is possible that failing septic tanks could be a contributor.  Oconee County continues to partner with OconeeWaters, a local environmental group which is part of the Upper Oconee Watershed Network (UOWN OconeeWaters) to continue previously developed educational programs.  The partnership has developed “Storm Water Journey” a geocache trail hosted on the County’s Storm Water Management web page and “Protect our Water”, an educational outreach program originally developed in 2019 to target 2nd grade students and their families through the school system.  This program is also available digitally on the Oconee County Storm Water Management web page.

Tracking

The data collected by Water Resources during sampling is attached to this evaluation report.